TASK 11 – RECOMMEND AND DEFEND THE OPERATING-SYSTEM PLAN

1. Introduction

The smart hospital requires different operating-system environments because each device has different requirements for response time, reliability, performance, security and resource usage.

Using one general-purpose operating system for every device may cause delays in critical alarms, unnecessary resource consumption and system failures. Therefore, a combination of specialised operating systems is recommended.

The proposed design supports the current 500-bed hospital and future expansion to three hospitals.

2. Recommended Operating-System Plan

| Hospital Environment | Recommended OS Type | Recommended OS Structure | Justification |
| --- | --- | --- | --- |
| Bedside patient monitors | Hard Real-Time Operating System (RTOS) | Microkernel | Provides predictable response for critical alarms that must be raised within 10 milliseconds. |
| Nurses' handheld tablets | Handheld/Mobile OS | Layered or hybrid | Supports mobility, touchscreen applications, wireless communication and battery management. |
| Doctors' desktop workstations | Desktop OS | Hybrid | Supports clinical applications, multitasking, user interfaces and secure access to patient records. |
| Central patient database | Clustered OS | Modular kernel | Provides high availability, failover, storage management and continuous database services. |
| MRI imaging server | Multiprocessor OS | Modular monolithic kernel | Supports multiple processors and parallel processing of large MRI images. |
| Partner hospital communication | Distributed OS environment | Client-server and distributed architecture | Enables secure communication and |

3. OS Services, Protection and I/O Methods

| Environment | Required OS Services | Protection Mechanisms | I/O Method |
| --- | --- | --- | --- |
| Bedside monitors | Process execution, interrupt handling, error detection and communication | Memory protection, restricted privileges and secure access | Interrupt-driven I/O |
| Nurses' tablets | User interface, file management, communication and power management | Authentication, encryption and application permissions | Interrupt-driven I/O |
| Doctors' workstations | Process management, file management, networking and resource allocation | User authentication, access control and process isolation | Interrupt-driven I/O |
| Central database | File management, communication, resource allocation, error detection and accounting | Role-based access, encryption, auditing and backups | DMA for bulk data transfers and interrupt-driven I/O |
| MRI server | Process management, memory management and device management | Process isolation and controlled device access | DMA |
| Partner hospital link | Communication, networking and security services | TLS encryption, authentication and access control | Network interrupt-driven I/O |

Explanation

Bedside monitors:
A hard RTOS is recommended because the system must respond to abnormal patient readings within 10 milliseconds. Interrupt-driven I/O allows the processor to respond when a sensor event occurs instead of continuously checking the sensor.

Nurses' tablets:
A mobile operating system is suitable because it provides wireless communication, touchscreen support and battery management. Authentication and encryption protect patient information.

Doctors' workstations:
A desktop operating system provides a suitable environment for clinical applications and multitasking. Access control ensures that only authorised users can access patient records.

Central database:
A clustered system provides continuous availability. If one server fails, another server can take over database services. Backups and access control help protect patient information.

MRI imaging server:
A multiprocessor operating system allows multiple processors to work on MRI image-processing tasks. DMA reduces CPU involvement during large data transfers.

Partner hospital communication:
A distributed architecture allows the three hospitals to exchange records securely. Authentication and encrypted communication protect data during transfer.

4. Limitations and Risks of the Proposed Plan

| Limitation or Risk | Possible Effect | Recommended Solution |
| --- | --- | --- |
| Different operating systems | Increased administration and maintenance effort | Centralised system administration and standard configuration |
| RTOS implementation cost | Higher development and maintenance cost | Use RTOS only for critical monitoring devices |
| Cluster configuration complexity | Incorrect configuration may affect failover | Regular failover testing and monitoring |
| Network failure | Partner hospitals may temporarily lose access to shared records | Redundant network connections and local data availability |
| Cybersecurity attacks | Patient information may be exposed | Encryption, authentication, updates and security auditing |
| Hardware failure | Services may become unavailable | Redundant hardware, backups and disaster recovery |
| System updates | Updates may interrupt hospital services | Planned maintenance and testing before deployment |

The proposed plan improves reliability and performance, but it requires skilled administration, regular testing and security management.

5. Suitability for a Group of Three Hospitals

The proposed design remains suitable when the hospital expands to three hospitals.

Each hospital can maintain its own local monitoring systems, workstations and database services. A distributed architecture can connect the hospitals and allow authorised sharing of patient records.

The following measures are recommended:

- Each hospital should maintain local patient-monitoring services.

- Critical bedside alarms should continue to operate even if the network connection fails.

- Database servers should use clustering and replication to improve availability.

- Secure communication should be established between all three hospitals.

- Centralised identity and access management should control authorised users.

- Regular backups and disaster-recovery procedures should be maintained.

This design supports expansion without requiring all hospital operations to depend on a single central server.

6. Alternative Concept – Virtual Machines on Two Large Servers

If the entire hospital system were moved to virtual machines running on two large physical servers instead of using separate physical machines, the design would be different.

A hypervisor would run on each physical server and host multiple virtual machines. Each virtual machine could run a different operating system for a particular hospital environment.

Proposed Virtualisation Design

| Virtual Machine | Operating System / Purpose |
| --- | --- |
| VM 1 | RTOS environment for bedside monitoring |
| VM 2 | Mobile application backend |
| VM 3 | Desktop application services |
| VM 4 | Database server |
| VM 5 | MRI image-processing services |
| VM 6 | Partner hospital communication services |

Evaluation of Virtualisation

1. Response Time for Monitors

Virtualisation introduces additional processing overhead because the hypervisor manages access to CPU, memory and I/O resources.

For bedside monitors, this overhead may cause unpredictable delays. Therefore, ordinary virtual machines should not be trusted to meet the strict 10-millisecond alarm deadline without real-time virtualisation support and proper worst-case testing.

Dedicated physical hardware running a validated RTOS is the safer choice for critical bedside monitoring.

2. Memory and Processor Overhead

Virtual machines require memory for their guest operating systems and applications. The hypervisor also consumes CPU and memory resources.

If many virtual machines run simultaneously, they compete for available resources. This may reduce performance during heavy workloads such as MRI processing and database operations.

3. Reliability

Using only two physical servers creates a risk of a major service interruption if both servers fail or if their shared infrastructure becomes unavailable.

Although virtual machines can be moved or restarted on another host, virtualisation alone does not guarantee continuous availability. Redundant storage, power supplies, network connections and tested failover arrangements are also required.

Comparison of Both Approaches

| Factor | Separate Physical Systems | Virtual Machines on Two Servers |
| --- | --- | --- |
| Monitor response time | More predictable with dedicated RTOS hardware | May experience hypervisor and resource-scheduling delays |
| Resource usage | Dedicated resources | Additional memory and CPU overhead |
| Isolation | Hardware-level separation | Virtual machine isolation |
| Reliability | Failure can remain limited to one device or server | Failure of a physical host may affect several VMs |
| Administration | More physical machines to maintain | Centralised management |
| Scalability | Requires additional hardware | New VMs can be created more easily |
| Cost | Higher hardware and maintenance cost | Potentially lower hardware cost, but requires powerful servers |

7. Complexity Analysis

Let:

- n = number of hospital devices or processing requests.

- p = number of physical servers.

- v = number of virtual machines.

| Operation | Separate Physical Systems | Virtualised Systems |
| --- | --- | --- |
| Managing devices/VMs | O(n) | O(v) |
| Processing requests | Depends on workload | Depends on workload and resource allocation |
| Memory usage | Dedicated memory per physical system | Guest OS memory plus hypervisor overhead |
| Resource sharing | Limited sharing | CPU and memory shared among VMs |

Virtualisation can reduce the number of physical machines and simplify central administration. However, it introduces additional resource-management overhead. The actual performance difference must be determined through measurements.

8. Evidence from Previous Tasks

The proposed design is supported by the following analysis:

- Task 1: The bedside monitors require a response within 10 milliseconds, making predictable real-time operation essential.

- Task 2: Different operating-system types are suitable for real-time devices, mobile devices, desktop systems, multiprocessor servers and distributed environments.

- Task 3: Microkernel, modular and hybrid structures provide different advantages depending on the environment.

- Task 4: Security, communication, resource allocation and error detection are essential OS services.

- Task 5: System calls provide controlled access to operating-system services.

- Task 6: Increasing buffer size reduces the number of read system calls for file-copy operations.

- Task 7: Process creation and execution can be managed using fork(), execlp() and waitpid().

- Task 8: CPU modes, interrupts and memory protection help protect critical hospital operations.

- Task 9: DMA is suitable for transferring large MRI image data with less CPU involvement.

- Task 10: Measurements of system calls, execution time, process creation and thread creation help evaluate actual system performance.

The measured values from Task 10 should be included here when available. The expected system-call counts from Task 6 are not substitutes for actual execution measurements.

9. Final Recommendation

A combination of specialised operating systems is recommended for the smart hospital instead of using one general-purpose operating system everywhere.

A hard RTOS should be used for bedside monitors, mobile operating systems for nurses' tablets, desktop operating systems for doctors' workstations, clustered systems for the central database, multiprocessor systems for MRI processing and distributed architecture for communication between hospitals.

The use of dedicated hardware for critical bedside monitoring provides better control over response time. Clustering improves database availability, while DMA supports efficient MRI data transfers. Security mechanisms such as authentication, encryption, access control and auditing protect patient information.

For the expansion to three hospitals, the proposed design can be extended through secure distributed communication and database replication.

Although virtualisation provides easier administration and resource sharing, using only two physical servers for every hospital function introduces performance and reliability risks. Therefore, a hybrid approach using dedicated real-time hardware for critical monitoring and virtualisation for suitable non-critical services is recommended.

Thus, the proposed operating-system plan provides a balance between patient safety, performance, reliability, security, scalability and maintenance.